i2022 Quick Start Guide

Remember SAFETY FIRST.  Refer to the User Guide for complete Safety Instructions.

Consulting with your system administrator or service provider before attempting to set up your phone can save time. This contact may also provide the information you need (switch type, SPID, Directory Numbers, switch features, and local features) to set up the Teleset.

Refer to the User Guide for complete setup, installation and operating instructions.

- Carefully unpack the box and check for the following parts:

  Teleset			  Wall-Mount/Desk-Stand Base

  S Interface Module 		  U Interface Module (Optional)

  Data Module (Optional)	  Handset and Handset Cord

  Designation Card		  Designation Card Faceplate

  Handset Holder		  Mounting Cord (eight-pin, round)

- Remove the S Interface Module from the Teleset and set the PS1/PS2 switch to the PS2 position, then set the Terminating Resistor (TR) switch located on the bottom side of the Interface Module.  NOTE: There is no PS1/PS2 switch on the U Interface Module.

- Reinstall the Interface Module.

- Install the Optional Data Module, if so equipped.

- Install the base to the bottom of the Teleset.  The base can be installed as either a wall-mount or a desk stand.

- Connect one end of the handset cord to the handset, and the other end to the handset jack on the left side of the Teleset. The handset jack is the rear most jack on the left side of the phone.

- Connect one end of the eight-pin mounting cord to the Line-In jack on the Rear of the Teleset. Connect the other end of the cord to the S/T jack on your NT1 (S Interface Module) or to the Power Adapter (U Interface Module).

- Remove the protective film on the display and install the logo plate.

- Attach the handset and headset holders if required.

- The i2022 will now attempt to Auto-Install.  The Teleset will “walk” you through all of the options.  You may abort the Auto-Install by pressing the Menu key at any time.

# AUTO-INSTALL

The purpose of Auto-Install is to ensure a complete and painless installation.  The i2022 will step through each of its programming choices and give you the opportunity to make a selection.  After you have made the selection press the “MORE” key to continue.

The Auto-Install choices are:

## Adjust Contrast – Select Yes or No

- SPID – If the switch supports Auto-SPID you will be prompted to choose a SPID.  If your switch does not support Auto-SPID, press the “Change” softkey and enter the SPID.  After entering the SPID press “Save”.

- The Teleset will attempt to initialize.  If there is a problem the Teleset will prompt you to correct the SPID and/or switch type.

- The Teleset will download its configuration from the switch.  If Parameter Downloading is not available, you will be prompted to program the Teleset manually.

- Backlight – Select On, Off, or Dynamic.

- Enter the Date and Time.

- Adjust the Ring Pattern – Select Yes or No.

- Set the 2nd ring Option – Select Continuous, Partial, or No Ring.

- Set Primary Call Appearances.

- Set Priority Call Appearances.

- Set Terminate Only Call Appearance.

- Set an Intercom Key.

- Set Speed Dial Keys.

- Set the Alarm Clock.

- Set a Password.

Your Teleset is configured and should start working.  Install the Designation card and Faceplate.

004-LUC-I0007-B